BUILDERS PACK

LINEAR HIGH BAY

ST36 SERIES

DESCRIPTION:
•  High luminous efficacy 140lm/w;

•  High quality non-isolated driver; 1-10V dimming; Flicker.

•  PF >0.90, driver efficiency >87%; THD<20%

•  LED Linear high bay lighting is a popular choice in warehouse lighting.

•  They are easy to hang with two gribber cables and easy to connect with the junction box right on

back.

•  Popular for their wide light distribution, they are a great choice for going LED and saving energy.

APPLICATIONS:
•  Designed for warehouses, gymnasiums, and industrial spaces;

SPECIFICATIONS

| SKU | Model | Wattage | Size | CCT | Efficacy | Lumens | Input Current |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 25-31170-xx | ST36-LHB-12-7110-8XX-B | 110W | 1x2' | 4000K | 140 LPW | 15400 lm | Max. 1.10A |
|  |  |  |  | 5000K |  |  |  |
| 25-31171-xx | ST36-LHB-12-7165-8XX-B | 165W |  | 4000K |  | 23100 lm | Max. 1.65A |
|  |  |  |  | 5000K |  |  |  |
| 25-31203-xx | ST36-LHB-14-7225-8XX-B | 225W |  | 4000K |  | 31500 lm | Max. 2.25A |
|  |  |  |  | 5000K |  |  |  |
| 25-31172-xx | ST36-LHB-14-7225-8XX-B | 225W | 1x4' | 4000K |  | 31500 lm | Max. 2.25A |
|  |  |  |  | 5000K |  |  |  |
| 25-31173-xx | ST36-LHB-14-7320-8XX-B | 320W |  | 4000K |  | 44800 lm | Max. 3.20A |
|  |  |  |  | 5000K |  |  |  |

| Photomeritc |  |  |  |  |
| --- | --- | --- | --- | --- |
| CRI | SDCM | R9 | Duv | Beam Angle |
| ≥80 | ≤4 | ≥0 | ≤0.006 | 90° |

| Electrical |  |  |
| --- | --- | --- |
| Frequency | Voltage | Power Factor |
| 50/60Hz | AC 120-277V | ≥0.90 |

| Lifetime [at 120-227V] |  |  |
| --- | --- | --- |
| L70 Lumen Maintenance Life [Hour] | L90 Lumen Maintenance Life [Hour] | Warranty [Years] |
| ≥50000 | ≥36000 | 5 |

| Technical Requirements |  |
| --- | --- |
| Certifications | UL I CE I FCC I RoHS |
| THD | ≤20% |
| Ripple Current | ≤5% |
| Frequency of LED Current | ≥120Hz |
| Flicker | Flicker |
| Electrostatic Discharge [ESD] | 4KV Contact discharge / 8KV Air discharge |
| Surge Protection | 4KV |
| Dimming | 1-10V |
| Sensor | Optional |
| Emergency Power | Optional |
| IP Degree | IP20 |

| Operation and Information |  |
| --- | --- |
| Start Time | ≤1 S |
| Operating Temp. | -4℉~+113℉ |
| Storage Temp. | -22℉~+149℉ |
| Housing / Case [Color, Material] | White, Steel |
| Optical - Lens [Material] | PC |
| Mounting / Installation | Pendant Mount / Surface Mount |
| Environment | Indoor / Suitable for damp location |

LAMP CODE DEFINITIONS

| Internal Use | Product Code | Size | Wattage | CCT | Version |
| --- | --- | --- | --- | --- | --- |
| ST36 | LHB | 12 = 1x2' | 110W | 8XX = 4000K, 4500K, 5000K, 5700K | B |
|  |  |  | 165W |  |  |
|  |  |  | 225W |  |  |
|  |  | 14 = 1x4' | 225W |  |  |
|  |  |  | 320W |  |  |

DIMENSIONS

1x2' 110W

1x2' 165W

1x2' 225W

1x4' 225W

1x4' 320W

| Product Size | Wattage | Long (L) | Wide (W) | High (H) | Net Weight |
| --- | --- | --- | --- | --- | --- |
| 1x2' | 110W | 23.82'' | 12.60'' | 2.99'' | 10.14 lbs |
|  | 165W | 23.82'' | 17.32'' | 2.99'' | 13.23 lbs |
|  | 225W | 23.64’’ | 14.17’’ | 2.89’’ | 17.22lbs |
| 1x4' | 225W | 45.87'' | 12.60'' | 2.99'' | 7.22 lbs |
|  | 320W | 45.87'' | 17.32'' | 2.99'' | 22.49 lbs |

PACKAGE

| Product Size | Wattage | Pcs / Carton | Master Carton Size | Gross Weight | Pcs / Pallet |
| --- | --- | --- | --- | --- | --- |
| 1x2' | 110W | 2 pcs | 25.78'' x 8.66'' x 15.75'' | 24.25 lbs | 80 pcs |
|  | 165W | 2 pcs | 25.78'' x 8.66'' x 20.47'' | 31.42 lbs | 64 pcs |
|  | 225W | 2 pcs |  |  |  |
| 1x4' | 225W | 2 pcs | 47.83'' x 8.66'' x 15.75'' | 40.12 lbs | 40 pcs |
|  | 320W | 2 pcs | 47.83'' x 8.66'' x 20.47'' | 49.38 lbs | 32 pcs |

*Package specifications are subject to change without notic

CONTAINER LOADING

20GP Container                                                  40GP Container

| 110w | 880pcs (No Pallet) ¦ 640pcs (Palletized) |  | 110w | 1980pcs (No Pallet) ¦ 1440pcs (Palletized) |
| --- | --- | --- | --- | --- |
| 165w | 704pcs (No Pallet) ¦ 512pcs (Palletized) |  | 165w | 1584pcs (No Pallet) ¦ 1152pcs (Palletized) |
| 1’x2’ 225w | 840pcs (No Pallet) ¦ 480pcs (Palletized) |  | 1’x2’ 225w | 750pcs (No Pallet) ¦ 1080pcs (Palletized) |
| 1’x4’ 225w | 460pcs (No Pallet) ¦ 320pcs (Palletized) |  | 1’x4’ 225w | 1010pcs (No Pallet) ¦ 720pcs (Palletized) |
| 320w | 368pcs (No Pallet) ¦ 256pcs (Palletized) |  | 320w | 808pcs (No Pallet) ¦ 576pcs (Palletized) |

40HQ Container                                                  45HQ Container

| 110w | 2340pcs (No Pallet) ¦ 1440pcs (Palletized) |  | 110w | 2600pcs (No Pallet) ¦ 1600pcs (Palletized) |
| --- | --- | --- | --- | --- |
| 165w | 1800pcs (No Pallet) ¦ 1152pcs (Palletized) |  | 165w | 2000pcs (No Pallet) ¦ 1280pcs (Palletized) |
| 1’x2’ 225w | 2100pcs (No Pallet) ¦ 480pcs (Palletized) |  | 1’x2’ 225w | 2400pcs (No Pallet) ¦ 1200pcs (Palletized) |
| 1’x4’ 225w | 1170pcs (No Pallet) ¦ 720pcs (Palletized) |  | 1’x4’ 225w | 1430pcs (No Pallet) ¦ 800pcs (Palletized) |
| 320w | 900pcs (No Pallet) ¦ 576pcs (Palletized) |  | 320w | 1100pcs (No Pallet) ¦ 640pcs (Palletized) |

AVAILABLE OPTIONS

| SKU | Description | Image |
| --- | --- | --- |
| 39-93011-A | PIR Sensor Use for Low Altitude |  |
| 39-93013-A | PIR Sensor Use for High Altitude |  |
| 39-93017-A | Microwave Sensor Use for Low Altitude |  |
| 39-93019-A | Microwave Sensor Use for High Altitude |  |
| 37-31175-A | Surface Mount |  |
| 37-31176-A | Wire guard use 110W |  |
| 37-31177-A | Wire guard use 165W |  |
| 37-31178-A | Wire guard use 225W |  |
| 37-31179-A | Wire guard use 320W |  |
| 78-20178-A | Remote Control with Standby Functions and Digital Display |  |
| 44-33005-A | Remote Control with Standby Functions |  |
| 44-33008-A | Remote control On/Off Only |  |
| 58-92057 | Emergency Power 15w, for 110w/165w LHB |  |
| 58-92058 | Emergency Power 20w, for 225w LHB |  |
| 58-92059 | Emergency Power 25w, for 320w LHB |  |